Your project already contains a surprisingly complete video-to-geospatial-3D pipeline:

- Geometry-aware video keyframe selection

- GPS/telemetry synchronization and uncertainty

- Dynamic-object masking

- COLMAP incremental, pose-prior and global reconstruction

- Dense point-cloud and mesh generation

- GCP correction

- DSM, LAS, PLY, GLB and OBJ export

- Runtime, GPU, reconstruction and checkpoint validation

- Browser-based metric viewer

The main weakness is no longer “can it reconstruct a scene?” It can. The weakness is proving that the resulting surface is accurate, complete and repeatable.

Your strongest potential contribution is:

A time-bounded, uncertainty-aware drone-video reconstruction system that produces auditable geospatial products and refuses to claim accuracy without independent evidence.

What the project currently uses

| Pipeline area | Current implementation | Assessment |
| --- | --- | --- |
| Keyframes | Sharpness, GPS baseline and optical-flow parallax | Good and relevant for video |
| Dynamic masking | YOLOv8 segmentation per selected frame | Useful, but not temporally consistent |
| Feature matching | COLMAP SIFT with sequential and optional spatial matching | Reliable baseline, but newer matching options exist |
| Sparse reconstruction | Incremental, pose-prior and global mapper options | Strong foundation |
| Georeferencing | GPS camera priors, ENU/UTM conversion | Good |
| Ground control | Robust 3D similarity transformation before export | Helpful, but does not correct local deformation |
| Dense reconstruction | COLMAP PatchMatch Stereo and fusion | Proven and sensible default |
| Meshing | Poisson or Delaunay, followed by cleanup | Good, but another native mesher should be tested |
| Accuracy | GPS residual, trajectory error, known distances and surveyed checkpoints | Good start; checkpoint protocol needs upgrading |
| Confidence | Point-density-based confidence raster | Only a coverage proxy, not true uncertainty |
| Products | DSM, LAS, PLY, GLB, OBJ and viewer | Strong; orthomosaic, DTM, LAZ and COG are missing |
| Reproducibility | Broad dependency ranges | Versions need to be locked |

Relevant implementation locations include [keyframe extraction (line 79)](D:/sih_drone_pipeline/SIH26158/sih_drone_pipeline/extract_keyframes.py:79), [AI masking (line 15)](D:/sih_drone_pipeline/SIH26158/sih_drone_pipeline/ai_masking.py:15), [COLMAP orchestration (line 337)](D:/sih_drone_pipeline/SIH26158/sih_drone_pipeline/colmap_pipeline.py:337), [checkpoint validation (line 113)](D:/sih_drone_pipeline/SIH26158/sih_drone_pipeline/validation.py:113), and [GIS export (line 302)](D:/sih_drone_pipeline/SIH26158/sih_drone_pipeline/export_gis.py:302).